Batı Anadolu Neojen Havzalarında Jeokimyasal Prospeksiyonun Önemi:

Emet (Kütahya) Borat Havzasında Hedef Saptama Örneği

Selçuk Tokel1, Cafer Özkul2 ve Mehmet Savaş3

1 Kocaeli Üniversitesi, Jeoloji Mühendisliği Bölümü, 41380 Kocaeli

2 Dumlupınar Üniversitesi, Jeoloji Mühendisliği Bölümü, 43100 Kütahya

(E-posta: caferozkul@yahoo.com)

3 Eti Maden, Emet Bor İşletme Müdürlüğü, 43700 Kütahya

Batı Anadolu’da geniş alanlar kapsayan karasal Neojen havzaları ekonomik önemde cevher birikimleri oluşturan evaporitik tuzlarca zengindir. Bu elementlerin çarpışma kökenli magmatizma ile ilgili hidrotermal sistemler yardımıyla, yan kayaçlardan çözündürüldüğü düşünülebilir. Cevherleşmeler bu termal suların sığ playa göllerine boşalıp buharlaşmasıyla oluşmuşlardır.

Bu havzalar içinde, bilinen merceklerin dışında, daha birçok mineralizasyon merceğinin varlığı düşünülebilir. Bunların ortaya çıkarılmasında jeokimyasal toprak prospeksiyonu uygulanabilir. Bu amaçla iki bilinen mineralizasyon merceği bulunan Emet-Hisarcık (Kütahya) Neojen havzası prospeksiyon için seçilmiştir.

Yaklaşık 400 km2 lik alan kapsayan örtü kayaçlarda 1 km lik aralıklarla oluşturulan grid üzerinde 370 adet toprakta B, As, Sn, Se, Li, Be, Mg, Sr, Na, K ve S analizleri yapılmıştır. Çözelti için sıcak A.R. kullanılmış, son ölçümler ICP-MS teknolojisi ile yapılmıştır.

Veri işlem ve istatistik çalışmalarıyla yapılan yorumlamalardan sonra toprak anomalileri ile bilinen gömülü mineralizasyonların boyut ve bileşimleri karşılaştırılmış ve şu sonuçlar ortaya çıkmıştır; (I) Bor prospeksiyonu için B elementi en uygun indikatör elementtir. Anomalinin şiddeti ve şekli bilinen mercekle tam uyumludur. (II) B anomalisi ile birlikte yer almış Na anomalileri (temel değerin 30 katında) mercekte kolemanit ile birlikte bulunan Na lu boratların (üleksit, probertit) varlığını işaret etmektedir. (III) B anomalileriyle birlikte görülen yüksek As değerleri mineralizasyondaki realgar ve orpiment miktarlarının derecesi için bir göstergedir. (IV) B anomalileri dışında çok keskin As ve Sb anomalilerinin varlığı ekonomik değerde stibinit birikimini işaret edebilir. (V) K, Li, Cs ve Be anomalilerinin ortak sınırları, havza içinde dar bir oluk içinde çökelmiş boratlı killerin sınırları ile uyumludur.

Sonuç olarak, sondajların yüksek maliyeti düşünülürse jeokimyasal toprak prospeksiyonu ile hedef saptandıktan sonra sondaj programlanmalıdır.

Anahtar Sözcükler: toprak jeokimyası, bor prospeksiyonu, Emet borat havzası.

Importance Of Geochemical Prospection on the W Anatolian Neogene Basins:

An Example of Target Definition for Borate Deposits

in the Emet (Kütahya) Basin

Selçuk Tokel1, Cafer Özkul2 & Mehmet Savaş3

1 University of Kocaeli, Department of Geological Engineering, TR41380 Kocaeli, Turkey

2 University of Dumlupınar, Department of Geological Engineering, TR43100 Kütahya, Turkey

(E-mail: caferozkul@yahoo.com)

3 Eti Mine, Boron Business Enterprise Directorate, TR43700 Kütahya, Turkey

Continental Neogene basins, which cover wide areas in W. Anatolia, are rich in evaporate salts of economic interest to form orebodies. The elements are assumed to be derived from leaching of the surrounding rocks by hydrothermal systems associated with the syn-collisional magmatism. These deposits formed when the spring waters evaporated after flowing into playa lakes.

In these basins, beyond the known deposits, existance of many unknown mineralization lenses can be thought. Geochemical soil survey may be aplicable in order to explore new findings. For this purpose Emet-Hisarcık (Kütahya) Neogene basin, which contain two known borate lenses were chosen for this aplication.

Square grid of samples taken at 1 km intervals were used on the cover rocks of approximately 400 km2. Total 370 soil sample were analysed for B, As, Sn, Se, Li, Be, Mg, Sr, Na, K and S. Decomposition were made by A.R. solution. Estimation conducted by ICP-MS.

After data handling and statistical interpretations, relation between form and magnitute of the soil anomalies, shape and composition of the known underlying mineralization were compared. After this stage the results demonstrated that; (I) B is a most suitable indicator element for boron prospection. Magnitude and shape of the anomalies are perfectly consistent with the known borate lenses. (II) B anomalies together with the Na anomalies (up to 30 times higher than normal) indicate presence of higher amount of Na borates (ulexite, probertite) together with colemanite. (III) B anomalies with the high As values indicate the degree of presence of realgar and orpiment in the borate deposits. (IV) Very sharp As and Sb anomalies away from B anomalies indicate economic stibinite depositions. (V) K, Li, Cs and Be concentrations maps indicate the limits of the narrow basin in which borate bearing clayey formation deposited.

It can be concluded that inasmuch as drilling is the most expensive method target definition by soil geochemistry should be established than the drilling oriented.

Key Words: soil geochemistry, borate prospection, Emet borate basin.