Mersin-Tarsus Arasındaki Bölge Topraklarında ve Yeraltı Sularında Nitrat

ve Nitrit Düzeylerinin Karşılaştırılması

Mehmet Ali Kurt, Cüneyt Güler, Musa Alpaslan ve Can Akbulut

Mersin Üniversitesi, Mühendislik Fakültesi, Jeoloji Mühendisliği Bölümü, 33343 Çiftlikköy, Mersin

(E-posta: malikurt@mersin.edu.tr)

Mersin ilinin doğusunda yer alan inceleme alanı; tarımsal (özellikle seracılık) ve endüstriyel faaliyetlerin birlikte ve yoğun olarak görüldüğü bir alandır. Bu çalışma ile bölgedeki topraklarda ve yeraltı sularındaki nitrat ve nitrit düzeylerinin belirlenmesi ve alansal dağılımının Coğrafi Bilgi Sistemi (CBS) yardımıyla belirlenmesi ve potansiyel kirletici kaynaklarının ortaya konulması amaçlanmıştır. Bu amaçla, Mersin-Tarsus arasında kalan bölgeden Ağustos 2007’de 208 toprak örneği (0–15 cm derinlikten) ve 157 yeraltı suyu örneği alınmıştır. Toprak örneklerinden nitrat ve nitritin ekstraksiyonu 2,0 N KCl çözeltisi ile yapılmış olup, elde edilen sıvının ve alınan su örneklerinin nitrat ve nitrit içerikleri spektrofotometre ile belirlenmiştir. Elde edilen sonuçlara göre; bölgedeki topraklarda nitrat değerleri 10–1478 mg/L arasında değişirken; nitrit değerleri 0,0–9,6 mg/L arasında değişmektedir. Yeraltı sularında ise nitrat değerleri 0–1834 mg/L aralığında değişirken; nitrit değerleri 0,0–10,3 mg/L arasında değişmektedir. Bölgedeki toprak ve yeraltı sularındaki nitrat ve nitrit dağılımlarının CBS yardımıyla yapılan karşılaştırması sonucunda bu dağılımların birbirlerine benzerlik göstermedikleri saptanmıştır. CBS ortamında üretilen çeşitli tematik haritalar incelendiğinde; yeraltı sularındaki yüksek nitrat ve nitrit konsantrasyonlarının daha çok yerleşim alanları ve sanayi faaliyetlerinin yoğun olduğu bölgelerde görüldüğü; topraklardaki yüksek nitrat ve nitrit konsantrasyonlarının ise tarımsal faaliyetlerin yoğun olduğu bölgelerde görüldüğü belirlenmiştir.

Anahtar Sözcükler: Nitrat, nitrit, toprak, su, Deliçay, Tarsus Çayı, Coğrafi Bilgi Sistemi.

Comparison of the Nitrate and Nitrite Levels in Soils and Ground Waters of the Area Between Mersin and Tarsus

Mehmet Ali Kurt, Cüneyt Güler, Musa Alpaslan & Can Akbulut

Mersin Üniversitesi, Mühendislik Fakültesi, Jeoloji Mühendisliği Bölümü, Çiftlikköy,

TR33343 Mersin, Turkey (E-mail: malikurt@mersin.edu.tr)

Study area is located east of the city of Mersin where both agricultural (especially greenhouse cultivation) and industrial activities are very intense. Aims of this study were; to determine nitrate and nitrite levels in soil and ground water samples collected from the area and their spatial distributions using a Geographic Information System (GIS); and to find out their potential sources of pollution. With this purpose, 208 topsoil samples (from a depth of 0–15 cm) and 157 ground water samples were collected in August 2007 from the area between Mersin-Tarsus. Extraction of nitrate and nitrite from soil samples was carried out using a 2.0 N KCl solution and nitrate and nitrite concentrations of obtained extracts and collected ground water samples were determined by spectrophotometer. According to results obtained from this study; nitrate concentrations in soil samples range between 10–1478 mg/L; whereas nitrite concentrations range between 0.0–9.6 mg/L. In ground water samples, however, nitrate concentrations range between 0–1834 mg/L; whereas nitrite concentrations range between 0.0–10.3 mg/L. Comparison of the spatial distribution of nitrate and nitrite in soils and ground water of the area using GIS revealed that their spatial distributions do not show a strong correlation. Examination of thematic maps produced by GIS indicate that high nitrate and nitrite concentrations in ground water generally occur around settlement areas and industrial facilities; whereas, high nitrate and nitrite concentrations in soils of the area generally occur in places where agricultural activities are very intense.

Key Words: Nitrate, nitrite, soil, water, Deliçay, Tarsus river, Geographic Information System.