Bir Enerji Havzası: Trakya Bölgesi

An Energy Basın: Thrace Regıon

İlker ŞENGÜLER*, Atilla SAKA*, Şafak AYDIN*, Serkan KARACA*, Cuma DAĞ*, Tamer TANSAK*, Ejder YAPICI*, Kadir NALKIRAN* ve Ferah KUVEL**

* MTA Genel Müdürlüğü Enerji Hammadde Etüt ve Arama Dairesi, 06520 Ankara

** MTA Marmara Bölge Müdürlüğü, 41001 Kocaeli

ilkersenguler@gmail.com

ÖZ

Trakya Bölgesi kömür ve hidrokarbon potansiyeli bakımından ülkemizin önemli havzalarından biridir. Bölgede kömür aramaları MTA tarafından, petrol ve doğalgaz aramaları ise genellikle TPAO tarafından yapılmıştır.

Trakya Tersiyer havzasında kömür oluşumları Oligosen yaşlı Danişmen Formasyonu içerisinde yer alır. Danişmen Formasyonu gri yeşil renkli kiltaşı, kumtaşı, çakıltaşı, tüf ve linyit içermekte olup bölgede yapılan ilk çalışmalarda linyitli kumtaşları olarak adlandırılmıştır.

Linyit oluşumları, Trakya Havzası’ nın kuzeyinde Istranca Dağlarının eteklerinde yer alır ve genellikle Saray (Edirköy, Safaalan, Küçükyoncalı), Vize (Topçuköy), Kırklareli, Demirhanlı sahaları olarak, güneyindeki linyit oluşumları ise Keşan, Malkara, Uzunköprü ve Meriç sahaları olarak anılır. 1950-1980 yılları arasında MTA Genel Müdürlüğü tarafından belirli derinlikte yapılan sondajlı aramalar sonucunda yaklaşık 500 milyon ton linyit rezervi tespit edilmiştir.

Havzanın kuzeyinde ve güneyinde yüzeyleyen linyitler, havzanın ortasına doğru tedrici olarak derinleşmekte ve havzanın orta kesimlerinde 10 000 m ye ulaşan çökel istif içerisinde, 600 m yi aşan derinliklerde yer almaktadır.

MTA Genel Müdürlüğü tarafından 2005 yılından bu yana bölgede yürütülen etüt ve sondaj çalışmaları ile havzanın derin kesimlerindeki kömür potansiyeli araştırılmaktadır. Havzada 300-500 m derinliklerde yer alan linyit oluşumları, havzanın linyit potansiyelini önemli oranda artıracaktır.

Anahtar Sözcükler: Trakya, Enerji, Kömür

ABSTRACT

Thrace Basin is one of the most important basins because of coal and hydrocarbon potential. Coal explorations have been conducted by General Directorate of Mineral Research and Exploration (MTA), oil and natural gas explorations usually by Turkish Petroleum Corporation (TPAO).

Coal formations in Thrace Basin of Tertiary are within Danişmen Formation of Oligocene. Danişmen Formation has a lithology including gray-green claystone, sandstone, conglomerate, tuff and lignite, and was named as lignitic sandstones in the region by first studies.

There are lignite occurrences on foothills of Istranca Mountains in N of Thrace Basin, and these are mentioned as Saray (Edirköy, Safaalan, Küçükyoncalı), Vize (Topçuköy), Kırklareli and Demirhanlı fields in general, and Keşan, Malkara, Uzunköprü and Meriç fields in the south. With drilled explorations at significant depths from 1950 to 1980 performed by General Directorate of Mineral Research and Exploration (MTA), about 500 million tons of lignite reserves were estimated.

Lignites exposed at N and S of the basin gradually deepen toward center of the basin, and reach to 10,000 meters of sedimentary sequence in central parts of the basin and extend to depths below 600 m.

Survey and drill studies conducted in the region since 2005 by General Directorate of MTA aim at exploring coal potential in deeper parts of the basin. Lignite occurrences at depths between 300 and 500 m in the basin will significantly increase lignite potential of the basin.

Key Words: Thrace, Energy, Coal