Gemlik Körfezi Yüzey Çökellerinde Petrol Aromatik

Hidrokarbonların Dağılımı ve Kaynaklarının Tanımı

Selma Ünlü ve Bedri Alpar

1 İstanbul Üniversitesi, Deniz Bilimleri ve İşletmeciliği Enstitüsü, Deniz Biyolojisi BD,

34116 Vefa, İstanbul (E-posta: su@istanbul.edu.tr)

2 İstanbul Üniversitesi, Deniz Bilimleri ve İşletmeciliği Enstitüsü, Deniz Jeolojisi ve Jeofiziği ABD, 34116 Vefa, İstanbul

Petrol aromatik hidrokarbon (PAH) bileşikleri başlıca doğal yollarla ve organik maddelerin tamamlanmamış yanmaları sonucu oluşan antropogenik süreçler ile denizel ortama katılırlar. Fosil yakıtların sızıntısı, evsel-endüstriyel atıkların deşarjı, atmosferik partiküllerin çökelmesi, egzoz gazlarının yoğunlaşması veya yüzey suları akışı gibi yollarla deniz ortamına giren PAH bileşiklerinin hafif molekül ağırlıklı olanlarının suda emisyon oluşturma özellikleri azdır ve bunların ağır moleküllü hidrokarbon bileşiklerine göre buharlaşma yetenekleri daha fazladır, diğer anlamda kolaylıkla biyolojik bozunuma uğrarlar. PAH bileşikleri bazen yerinde oluşan biyogenik bozuşum nedeniyle diyagenetik yollarla, bazen de petrogenik olarak oluşurlar. Biyo-jeokimyasal çalışmalarda PAH bileşiklerinin dağılımı yanında, olası kaynaklarının araştırılması ve bunların değişik ortam şartlarından nasıl etkilendiklerinin belirlenmesi de gerekmektedir.

Marmara Denizinin en kirli alanlarından biri olan Gemlik Körfezinden toplanan 62 adet yüzey çökel örneğindeki gaz kromatografisi-kütle spektrometresi ile ölçülen 13 adet PAH bileşiklerinin konsantrasyon artışları geniş bir spektrumda dağılım göstermektedir. Bu bileşiklerinin miktar tayinleri kirletici kaynak ayrımı için yeterli olmayıp, bu amaçla çeşitli oran, indis ve istatistiksel analizler kullanılır. Bu çalışmada Gemlik Körfezi güncel çökellerinde gözlenen PAH’ların temel kaynaklarını belirlemek ve doğal kaynakların insan kaynaklı kaynaklarla olan ayrımını yapabilmek için Varimax rotasyonu ile ana bileşenler analizi (PCA) uygulanmıştır. Aralarında yüksek ilgileşim bulunan çok değişkenli verileri aralarında ilgileşim olmayan yeni bir koordinat sistemine dönüştüren bu yöntem, kaynak türlerini önceden bilmeden sadece çok değişkenli giriş verilerini kullanarak regresyon analizi yapabilmektedir. Analiz sonucu elde edilen faktör skoru mutlak değerlerinin, ortalaması çıkarılıp standart sapmasına bölünmüş toplam PAH değerlerine yapılan çoklu doğrusal regresyon analizi ile kirletici ana kaynakların yüzde dağılımları belirlenmiştir.

Gemlik Körfezinde toplam PAH konsantrasyonları silt/kil yapısı ve organik karbon içeriği ile önemli korelasyonlar göstermez (r2=0.30 ve 0.18). Özellikle doğu ve güneydeki yakın-kıyı alanlarında yayılım gösteren yüksek değer dağılımlarının temel kaynakları, nehir girdileri, gemicilik ve liman faaliyetleri, yerel ve endüstriyel fabrika atığı deşarjları, hızlı ekoturizm ve atmosferik girdilerdir. Körfezin orta ve kuzey kesimlerindeki PAH bileşenlerinin karakteristik dağılımları daha farklı bulunmuştur. Körfez ortası kesimlerde fluoranthene ve pyrene baskın iken kuzey sahile yakın sahalarda perylene, Benzo[ghi]perylene ve indeno[l,2,3-cd]pyrene daha baskındır. Farklı PAH bileşiklerinin kaynaklarının neler olabileceği ve bunların birbirleriyle olan ilişkisini açıklayabilmek için bazı özel bileşen oranları değerlendirilmiştir. Perylenin penta-aromatik izomerler içindeki yüksek oranlarının özellikle kuzeyde ve körfezin derin kesimlerinde dağılması, kirletici kaynağın bu kesimde kısmen diagenetik bir orijine sahip olduğunu göstermektedir. Elde edilen veriler Gemlik Körfezi ve çevresindeki su havzasının çevre etki değerlendirilmesine katkıda bulunmakta, ayrıca PAH bileşiklerinin yarı kapalı denizel ortamlarda nasıl davrandığını anlamamıza olanak tanımaktadır.

### Anahtar Sözcükler: PAH, Gemlik Körfezi, sediment, istatistiksel analiz, kirlilik, kirletici kaynak

Distribution and Source Apportionment of Sediment PAHs

in Gulf of Gemlik, Sea of Marmara, Turkey

Selma Ünlü & Bedri Alpar

1 İstanbul University, Institute of Marine Sciences and Management,

Department of Marine Biology, Vefa, TR–34116 İstanbul, Turkey (E-mail: su@istanbul.edu.tr)

2 Istanbul University, Institute of Marine Sciences and Management,

Department of Marine Geology and Geophysics, Vefa, TR–34116 İstanbul, Turkey

Polycyclic aromatic hydrocarbons (PAHs) are a family of chemical compounds that are known to contain harmful substances, some of which may be carcinogenic. PAHs in marine sediments originate from three main sources. Diagenic and petrogenic PAHs were produced by biogenic precursors and by diagenic processes at low temperatures over geologic time scales, respectively. Pyrogenic PAHs result from incomplete combustion of organic substances including fossil fuels and biomass. Volcanic activity and forest fires, which do not significantly contribute to overall PAH emission, are natural sources of pyrogenic PAHs, while pyrogenic anthropogenic sources, which are mostly resulted from combustion of materials for energy supply and waste minimalization, contributes much to overall PAH emission in the environment. In closed and semi-closed environments, such as the Gulf of Gemlik in the Sea of Marmara, Turkey, the latest industrial and agricultural developments and rapid urbanization cause significant increase of PAH concentrations especially in sea bottom sediment. Elementary PAH inputs may be from stationary sources (e.g., industry, residential heating, power and heat generation) or from mobile sources (e.g., all kind of gasoline and diesel engines, incineration of municipal and industrial wastes and agricultural burning). Most of them can be transported over long distances before they are deposited in sediment.

Quantitative analyses showed that elevated concentrations of sediment PAHs in the Gulf of Gemlik demonstrate uneven distribution patterns throughout the gulf. But they show weak correlations with sediment texture and total organic carbon content, implying that there are direct pollution inputs affecting the sediment quality. In addition, the lighter PAH compounds are less hydrophobic and less persistent than the heavier ones. Therefore characteristic distribution of PAH species show some spatial discrepancies throughout the gulf; such that fluoranthene and pyrene are dominant in the central part while perylene, Benzo[ghi]perylene and indeno[l,2,3-cd]pyrene are dominant along the northern coastal areas. In order to identify the main sources of the sediment PAHs and to distinguish natural contamination from those arising from human activities, principal component analysis with Varimax rotation was employed. It is a useful tool for extracting regularities directly from the input data without referring to classes known in advance. The input data includes 13 parent PAHs of 62 bottom sediment samples from coastal and central parts of the gulf. Through Kriging analysis widespread contamination were identified along the eastern and southern shores possibly due to ports and harbors, fuel storage terminals and anchoring areas. Multiple linear regression analysis of the absolute factor scores against the standard normalized deviate of the sum of PAHs were used to quantify such kind of major source contributions. In one of the groups separated from others, total toxic B[a]P equivalency (TEQcarc) values were found elevated which poses considerable hazard to benthic organisms.

### Key Words: PAH, Gulf of Gemlik, sediment, statistical analysis, pollution, pollution source