Kazdağları’nın Kuzeybatısındaki Su Kaynakları

ve Bunları Etkileyen Çevresel Faktörler (Çanakkale-Türkiye)

Alper Baba1 ve Orhan Gündüz2

1Çanakkale Onsekiz Mart Üniversitesi, Jeoloji Mühendisliği Bölümü, Terzioğlu Kampusü,

17020 Çanakkale (E-posta: alperbaba@comu.edu.tr)

2Dokuz Eylül Üniversitesi, Çevre Mühendisliği Bölümü, 35160 Buca, İzmir

Kazdağları’nın kuzeybatı bölgelerini kapsayan inceleme alanı, geç Eosen’den başlayarak PliyoKuvaterner’e kadar çeşitli dönemlerde etkin olan volkanizmaların ürünü volkanoklastiklerle birlikte çökelmiş denizel, lagüner ve gölsel kırıntılı kayaçları içermektedir. Bu nedenle bölgede sedimanter ve hidrotermal kaynaklı çeşitli maden yatakları bulunmakta ve ekonomik olarak değerlendirilmektedir. Bu çalışma kapsamında yeraltısularının çevresel izotop içerikleri ve fiziksel özellikleri birlikte değerlendirilerek Kazdağları’nın kuzeybatısında bulunan soğuk su kaynaklarının birbirleri ile ilişkileri araştırılmıştır. Çevresel izotoplarla yapılan analizler sonucu soğuk su kaynaklarının beslenme kotları ve yeraltında kalış sürelerinin farklı olduğu ortaya konulmuştur. Su kaynaklarının jeolojik yapıya bağlı olarak farklı özellikler gösterdiği tespit edilmiştir. Altere olan zonlarda gelen kaynak sularında alüminyum (Al) değerleri içme suyu için önerilen limit değerlerini (0.2 ppm) oldukça aşmıştır. Bölgede çoğunlukla açık işletme olarak faaliyet gösterilen küçük ve orta ölçekli kömür sahalarının hiçbir rehabilitasyon önlemi alınmadan terk edilmesi neticesinde yüzeysel drenaj ve yeraltısuyu sızması sonucu iri ufaklı göller oluşmuştur. Söz konusu göller zaman içerisinde pirit oksidasyonuna bağlı olarak asidik maden göllerine dönüşmüşlerdir. Yüksek asidite (düşük pH: ortalama 2.83) ve yüksek derişimlerde çözünmüş metalleri (örneğin Al= 211 ppm, Fe= 528 ppm) içeren bu göller bölgedeki su kaynaklarını etkilemektedir.

Anahtar Sözcükler: Kazdağları, madencilik, asidik göller, pirit oksidasyonu, su kaynakları

Environmental Factors That Influence the Water Resources

of Northwestern Ida Mountains (Çanakkale-Turkey)

Alper Baba1 & Orhan Gündüz2

1 Çanakkale Onsekiz Mart Üniversitesi, Jeoloji Mühendisliği Bölümü, Terzioğlu Kampusü,

TR17020 Çanakkale, Turkey (E-mail: alperbaba@comu.edu.tr)

2 Dokuz Eylül Üniversitesi, Çevre Mühendisliği Bölümü, Buca, TR35160 İzmir, Türkiye

The study area that covers the northwestern slopes of Ida Mountains contains marine, lagoon and lake sediments deposited  with volcanoclastics that produced  as a result of the volcanism which was effective in various periods from the late Eocene to PlioQuaternary.  the region has several sedimentary and hydrothermal mineral deposits, which have en economic importance and already being mined. . Within the scope of this study, the environmental isotopic and physicochemical characteristics of groundwater samples were jointly assessed and mutual relationships between cold water resources found in northern west slopes of Ida Mountains were studied. The results of environmental isotopes revealed that the recharge zones and detention times of cold water resources were different. The water resources also exhibited distinct characteristics as a result of the interactions with local geological strata. Spring waters originating from altered formations were shown to contain elevated levels of aluminum that is higher than the currently effective drinking water quality limit value (0.2 ppm). In addition, the abandoned small to medium scale open-pit coal mines in the area were found to be responsible from the formation of several mining lakes that were inundated as a result surface drainage as well as subsurface infiltration to depression areas of the open pit. These lakes were later transformed into acidic mining lakes due to the oxidation of pyrite found in the coal. With very high acidity levels (pH levels as low as 23) and high trace element contents (with maximum aluminum and iron levels of 211 ppm and 528 ppm), these lakes were found to influence the quality of local water resources in the area.

Key Words: Ida Mountains, mining, acidic mining lake, pyrite oxidation, water sources