Kumbaba (Şile) Plajı'nda Kıyının Evrimi

Çağdaş Coşkun

İstanbul Üniversitesi, Fen Bilimleri Enstitüsü, 34452 Vezneciler, İstanbul

(E-posta: cagdasco@gmail.com)

Kumbaba Plajı, Şile (İstanbul) liman bölgesinin batı kısımında yer almakta ve şehir merkezine yakın olması sebebiyle kıyı turizmi açısından önem arz etmektedir. Bu amaçla plajın jeolojik evrimi kıyı mühendisliği prensipleri açısından incelemiştir. Plajın evriminde en büyük etken olan dalga etkisi, Şile Meteoroloji İstasyonu'nda 19862006 yılları arasında toplanmış olan rüzgâr verileri ve NATO Tu-Waves Projesi kapsamında Şile bölgesi dalga atlası kullanılarak rüzgâr/dalga istatistiği yapılmış ve dalga özellikleri belirlenmiştir. Bunlara ek olarak, İngiliz Donanması'nın uzun zamanlar boyunca Karadeniz'de toplamış olduğu deniz meteorolojisi verilerinden elde edilen analizin diğer iki çözümlemenin değerlendirmesini destekleyerek kıyı boyu katı madde taşınımı tespit edilmiştir. Jeolojik ve jeomorfolojik temel üzerine kurulan plaj evrimi, meteorolojik verilerin analizlerinin Kıyı Mühendisliği içerisindeki akıntı-dalga mekaniği ve katı madde taşınımı ile sentezlenmesi ve farklı zaman dilimleri içerisinde çekilmiş uydu görüntüleri ve hava fotoğrafları gibi uzaktan algılama öğelerinin katkısıyla inceleme nihayetine ermiştir. İnceleme CERC (SPM) yöntemi ile yapılmıştır. Yapılan net kıyı boyu katı madde miktarları; dalga atlasına göre doğu-batı yönünde 14,946.249 m3/yıl, meteorolojik verilerle yapılan feç analizi sonucunda ise batı-doğu yönünde 1,205.722 m3/yıl olarak belirlenmiştir. Bu farkın oluşmasında kullanılan analiz yöntemi, rüzgâr verilerinin toplandığı istasyonun ve dalga atlasının konumu önemli etkenlerdir.

Anahtar Sözcükler: Şile, evrim, jeoloji, jeomorfoloji, kıyı, sediment, taşınım

Coastal Evolution of Kumbaba Beach (Şile, Black Sea)

Çağdaş Coşkun

İstanbul Üniversitesi, Fen Bilimleri Enstitüsü, Vezneciler, TR34452 İstanbul, Türkiye

(E-mail: cagdasco@gmail.com)

Geological evolution of the coastal beaches in the Kumbaba Beach, located west of Sile (Istanbul) waterfront, has been reviewed in terms of coastal engineering principles. The biggest factor in the evolution of the beach is wave effect. Wind/wave statistics were made and wave characteristics determined by using some wave and wind data to estimate the wave effect. Wind data collected by the Sile Weather Station and the data in the Wave Atlas of the Şile region acquired by the NATO Tu-Waves Project were used. An analysis from the long-term Black Sea meteorological data of the British Navy was used to support the other two solutions to assess the long shore sediment transport. Geological and geomorphological basis of the beach evolution was concluded using the analysis of meteorological data for the Coastal Engineering in terms of the stream-wave mechanics and sediment transport and remote sensing images (satellite images and air photos) for different time periods Review was done using CERC (SPM) method. According to wave atlas, the amount of long shore sediment transport is 14,946.24 cubic meters per year in the east to west direction, whereas according to the wind data the amount from west to east is 1,205.722 cubic meters per year. The main factors contributing to the difference are use of the different analytical methods and the position of the meteorological stations used for the collection of wave atlas and the wind data.

Key Words: Şile, evolution, geology, geomorphology, coastal, sediment, transport