Deliçay-Tarsus Çayı (Mersin) Arasında Kalan Bölgedeki Yeraltı Sularının Tarımsal Sulama Suyu Kalitesi Açısından Değerlendirilmesi

Can Akbulut, Mehmet Ali Kurt, Cüneyt Güler ve Musa Alpaslan

Mersin Üniversitesi, Mühendislik Fakültesi, Jeoloji Mühendisliği Bölümü, 33343 Çiftlikköy, Mersin

(E-posta: canakbulut@mersin.edu.tr)

Bu çalışmada, tarımsal (özellikle seracılık) ve endüstriyel faaliyetlerin yoğun olduğu Deliçay-Tarsus Çayı (Mersin) arasında kalan bölgedeki yeraltı sularının tarımsal sulama suyu olarak kullanılabilirliği araştırılmıştır. Ağustos 2008’de bölgedeki mevcut kuyulardan seçilen toplam 215 kuyudan yeraltı suyu örnekleri alınmış ve çeşitli fiziksel parametrelere (elektrik iletkenlik ve pH) ait değerler, majör iyonlar (kalsiyum, magnezyum, sodyum ve potasyum) ve anyonların (sülfat, klorür, karbonat ve bikarbonat) konsantrasyonları belirlenmiştir. Elde edilen sonuçlara dayanarak, herbir su numunesi için sodyum yüzdesi (% Na) ve sodyum adsorbsiyon oranları (SAR) belirlenmiş, Wilcox ve ABD Tuzluluk Laboratuvarı diyagramları çizilmiş ve incelenen yeraltı sularının sulama suyu olarak kullanılabilirliği Su Kirliliği Kontrolü Yönetmeliği, Teknik Usuller Tebliği’nde belirtilen sulama suyu kalite kriterleri doğrultusunda değerlendirilmiştir. Buna göre; suların %23’ü II. sınıf (iyi) ve %76’sı III. sınıf (kullanılabilir) kalitede sular olarak belirlenmiştir. 153 nolu kuyudaki su IV. sınıf (ihtiyatla kullanılabilir) kalitede iken 47, 121 ve 195 nolu kuyulardaki suların V. sınıf sular olduğu ve bu kuyulardaki suların sulamada kullanılmasının sakıncalı olduğu belirlenmiştir.

Anahtar Sözcükler: tarımsal sulama suyu, yeraltı suyu, su kalitesi, Deliçay, Tarsus çayı, Mersin

Assessment of Ground Waters in the Area Between Deliçay-Tarsus River (Mersin) with Respect to Irrigation Water Quality

Can Akbulut, Mehmet Ali Kurt, Cüneyt Güler & Musa Alpaslan

Mersin Üniversitesi, Mühendislik Fakültesi, Jeoloji Mühendisliği Bölümü, Çiftlikköy,

TR33343 Mersin, Türkiye (E-mail: canakbulut@mersin.edu.tr)

In this study, ground waters in the area between Deliçay-Tarsus river (Mersin) were evaluated for their suitability for agricultural irrigation water where at present intense agricultural (especially greenhouse cultivation) and industrial activities are taking place. A total of 215 ground water samples were collected in August 2008 from the selected wells in the region and analyzed for various physical parameters (Electrical conductance and pH), major cations (calcium, magnesium, sodium and potassium) and anions (sulfate, chloride, carbonate and bicarbonate). Based on these results, for each water sample, percent sodium (Na %) and sodium adsorption ratio (SAR) values were determined, Wilcox and U.S. Salinity Laboratory diagrams were constructed and ground water samples were evaluated for their suitability according to criterions defined in Water Pollution Control Directive, Technical Methods Bulletin. According to results of this study; 23% of water samples are defined as Class II quality (good) and 76% of them as Class III quality (usable). Whereas, ground water from the well 153 is classified as Class IV quality (usable with caution) and ground waters from the wells 47, 121 and 195 are found to be Class V quality which indicates that waters from these wells are not suitable for irrigation.

Key Words: agricultural irrigation water, ground water, water quality, Deliçay, Tarsus river, Mersin.