Yüzey Manyetik Süseptibilite Ölçümleri İle Kirlilik Analizi

Ali Aydın, Mehmet Yılmaz ve Yusuf Balkan

Pamukkale Üniversitesi, Jeofizik Mühendisliği Bölümü, 20020 Kınıklı, Denizli

(E-posta: aaydin@pau.edu.tr)

Bu çalışmada daha önceki çalışmalarda manyetik süseptibilite ve ağır metal içerikleri arasında belirlenen ilişkiler kullanılarak, Denizli ili Bağbaşı beldesinde yer alan yollarının kenarları ve çevresinde yer alan yüzey toprağı içindeki kirlilik dağılımını gösterdik (Şekil 1). Eksoz gaz kirliliğinin çevreyi tehdit eden en önemli kirlilik kaynağı olduğunu ve ikinci olarak ta fabrika ve ev bacalarından çıkan dumanların olduğunu gösterdik. Bu amaçla, manyetik süseptibilite verisini kullanarak kirlilik dağılımını haritaladık ve çevreyi tehdit eden kaynakları gösterdik. Bu yöntemin kimyasal analiz yöntemlerine göre oldukça ucuz ve az zaman alıcı olduğunu ve de ağır metal dağılımını etkin bir şekilde ortaya koyduğunu gösterdik.

Yerleşim alanındaki ağır metal dağılımını 112 saha manyetik süseptibilite ölçüsü kullanarak göstermeye çalıştık. Bu ölçümler sayesinde eksoz gazları ve bacalardan çıkan baca dumanlarının çevreye bıraktığı ağır metal dağılımlarının yığılım yaptığı bölgeler belirlendi. Yerleşim alanlarındaki ağır metal kirlilik dağılımlarının ortaya konulmasında yalnız manyetik süseptibilitenin kullanılmasının yeterli olacağını gösterdik.

Şekil 1. Çalışma alanında sahada ölçülen manyetik süseptibilite ölçüm noktaları, manyetik süseptibilite, Cu ve Pb kontur haritaları

Anahtar Sözcükler: eksoz gazları, ağır metaller, manyetik süseptibilite, kirlilik

Pollution Analysis with Field Magnetic Susceptibility Measurements

Ali Aydın, Mehmet Yılmaz & Yusuf Balkan

Pamukkale Üniversitesi, Jeofizik Mühendisliği Bölümü, Kınıklı, TR–20020 Denizli, Turkey

(E-mail: aaydin@pau.edu.tr)

In this study, we used the relation between the magnetic susceptibility and the contents of heavy metals before studies and showed the distributions of the pollutions in the soil surface of around and sides of traffic roads in Bağbaşı District, Denizli (Figure 1). We saw the most important pollution sources that exhaust gases pollution is one of the most important environmental threats and second one is fumes from factory and houses. For this purpose, we did mapping of the pollutant distributions using the data of magnetic susceptibility and showed their sources to assess environmental threats. We showed that this method is cheaper and less time-consuming against chemical methods and also showing the dimensions of distribution of the heavy metal pollution.

Distributions of heavy metal pollution in the residential area were tried to show by using 112 fields magnetic susceptibility measurements. These measurements were made to find out the distribution of heavy metallic elements deposited from exhaust gases and fumes chimney in area near the high way and around.  We showed that it is enough only using field magnetic susceptibility measurements could provide heavy metal pollution distribution in residential areas.

Figure 1. The position of magnetic susceptibiliy measurements points, the countour maps of magnetic susceptibily, Cu and Pb

Key Words: exhaust gases, heavy metals, magnetic susceptibility, pollution