Tyana (Kemerhisar) Su Kemerlerinin Jeoarkeolojik Özellikleri

Mustafa Korkanç1 ve Sevil Tokgöz2

1 Niğde Üniversitesi, Jeoloji Mühendisliği Bölümü, 51290 Niğde

(E-posta: mkorkanc@nigde.edu.tr)

2 Niğde Üniversitesi, Fen Bilimleri Enstitüsü, 51290 Niğde

Niğde’nin 20 km güneyinde Adana-Kayseri yolunun 4 km kuzeyinde yer alan su kemerleri, Roma Havuzunun devamı niteliğindedir. Antik Roma Havuzunun suyunu taşıyan kemerler, Roma İmparatorları Trajen ve Hadriyan tarafından yapılmıştır. Yaklaşık 3 km uzunluğundaki kemerlerin 1,5 km’lik bölümü açıkta, gerisi toprak altında olup, kullanılamaz durumdadır. Niğde yöresinde geçmişten günümüze kadar, değişik medeniyetler tarafından inşa edilmiş çeşitli kültürel varlıklar bulunmaktadır. Oldukça zengin kültürel yapılara sahip olan bölgede, böyle bir çalışmanın yapılması ile bu yapıların oturduğu zeminle ilişkisi ve yapılarda kullanılmış olan taşlara ilişkin jeolojik sorunlar ortaya konularak, bu alanların korunması ve yapının gelecek nesillere aktarılması konusunda önemli bir amaca hizmet edeceği düşünülmektedir. Kemerlerde yapı taşı olarak travertenler kullanılmış olup, burada kullanılan yapı taşları gözlemsel olarak incelendikten sonra, bunların alındığı olası taş ocakları da belirlenmeye çalışılmıştır. İncelenen travertenlerin kısa mesafede, yanal ve düşey yönde sıkça değişen karmaşık bir iç yapıya sahip olmaları nedeniyle, yapıda kullanılmış olanlardan ve taş ocaklarından toplam 10 örnek derlemesi yapılmıştır. Derlenen bu örneklerden elde edilen veriler sonucunda ince taneli, masif dokulu ve boşlukları az olan örneklerin nispeten daha yüksek dayanım değerleri sundukları belirlenmiştir. Kullanılan travertenlerde ayrışma etkileri oldukça düşük olup, kemerlerin tahrip olmasında; insani etkiler yanında, taş boyutunun, özellikle kemerlerde kullanılan kilit taşlarının küçük boyutta ve aynı taş ocağında olmasına rağmen farklı ve kötü özellikler gösteren kesimlerden taş seçilmesinin etkin olduğu düşünülmektedir.

Anahtar Sözcükler: Niğde, jeoarkeoloji, su kemerleri, ayrışma, jeomekanik özellikler, traverten

Geoarchaeological Properties of The (Tyana) Kemerhisar Aqueducts

Mustafa Korkanç1 & Sevil Tokgöz2

1 Niğde Üniversitesi, Jeoloji Mühendisliği Bölümü, TR51290 Niğde, Türkiye

(E-mail: mkorkanc@nigde.edu.tr)

2 Niğde Üniversitesi, Fen Bilimleri Enstitüsü, TR51290 Niğde, Türkiye

Aqueducts situated 20 km South of Niğde and 4 km north of the Adana-Kayseri road are continuation of the Roma Pool. The aqueduct that carries the antique Roma Pool’s water was made by the Roman Emperors Trajen and Hadriyen. 1.5 km part of approximately 3 km long aqueducts is apparent, the rest of it is under ground and it is useless. In Niğde region, various cultural assets were built by different civilizations. With this kind of study which will be done in this region that has quite rich cultural buildings, it is considered that the relations of these buildings with the ground on which these buildings rest and the geological problems related to the stones used in these buildings will be exposed and in the subject of conservation of these regions and transmitting these buildings to the next generations, serve an important aim. In aqueducts, travertines were used as a building stone and after the investigation of building stones used for these buildings, the quarries that they were taken from were tried to be determined. Because of having complex inner structure changing frequently in horizontal and vertical directions in short distances, 10 travertine samples were collected from building and quarries. As a result of the obtained data, it is determined that fine grained, massive textured and low porous samples show relatively higher strength values. Weathering effects on travertines are quite low and in destruction of aqueducts, it is thought that, beside human effects, stone dimensions, rock usage and selection from the same quarry and from different places showing different and bad properties are also effective.

Key Words: Niğde, geoarchaeology, aqueducts, weathering, geomechanical properties, travertine