Using Wavelet Transform for Fusion Landsat7-(ETM+) Panchromatic and Multispectral Images Data.

Abd-alrahman Ramzi

Remote sensing centerMosul University

Iraq

abdfreezone@yahoo.com

ABSTRACT

Imagery plays a major role in a diverse number of fields such as robotics, urban planning medical imaging , environmental monitoring, and strategic planning .Image fusion is a technique used to integrate the geometric detail of a high-resolution panchromatic(PAN) image and the color information of a low-resolution multispectral (MS) image to produce a high-resolution MS image .The goal of image fusion is to integrate complementary information from multisensor data such that the new image are more suitable for the purpose of human visual perception and computer-processing tasks such as segmentation ,feature extraction ,Classification, and object recognition. Most earth resource satellites, such as SPOT,IRS,LANDSAT 7 ,IKONOS, QuikBird and OrbView, provide both Pan image at a higher spatial resolution and MS images at a lower spatial resolution. An effective image fusion technique can virtually extend the application potential of such remotely sensed images, as many remote sensing applications require both high-spatial and high-spectral resolution, especially for GIS based application.

In this paper we use the technique based on wavelet transform, wavelet decomposition is used for image processing in wide range. Wavelet transform produce the images in different resolution .Wavelet representation refers to both spatial and frequency space. It can show a good position of a function(here this function is the image) in spatial and frequency space. there are different approaches to do wavelet decomposition. One of them is Mallat algorithm  which can use wavelet function such as “Daubechies function” ,”(db1,db2,….”.

Merging information from different imaging sensors involves two distinct steps. first the both image mast be geometrically registered to one another. Next the information contents- both spatial and spectral were mixed to great a single data set that contained both spatial and spectral were mixed to generate a single data set that contained the best of both sets. Two steps are necessary for the wavelet merging method, first step ,having produced three new ETM (Pan) images whose histograms are specified according to the histogram of each ETM+ (MS) band. after wavelet transform for each( pan) images , each set of three detail images extracted from one (pan) image corresponds to the spatial information, 15-m for one ETM+  (MS) band. The second step consisted of introduction these details into each (MS) band through the inverse wavelet transform. Simulated images are produced. The spectral information content of original (MS) images was conserved.

To quantify the behavior of the fusion methods used here, we computed the correlation between the images result by suggestion method and the original multispectral image. High of the correlation shows that the spectral characteristic of the multispectral image has been preserved well .

The major advantage of the wavelet merging method is in the minimal distortion of the spectral characteristics of the data. This result could be obtained because the wavelet method has only added the scale structures contained between 15-m resolution and 30-m resolution images. Another advantage was that these scale structures ((pan) details) can be applied to each (MS) band independently. The results stemming from this merging method are very promising in the field of Analysis and thematic processing (i.e., production of thematic color composite images ,automatic classification…).

We use the landsat 7 ETM+ image data which have been collected on 2001 . It is ETM+ (MS) and ETM+ (Pan) Image of the mosul dam on the river “Tigris” north mosul city in Iraq .