Geological Study for the Southern plain of Alan/ Northwest of Mosul Using Geoelectrical Sounding

- Ali Saadeldeen Abbas Al-Mwaly

- Dr. Marwan mutib

Soil & Water Science Dept., College of Agriculture and Forestry, Mosul University

Geology Dept., College of Science, Mosul University

E-mail: aliturkey2000@yahoo.com

ABSTRACT

The present research involves geoelectrical study of the Southern Allan’s plain, which lies between Allan and Atshan anticlines, in the North of Iraq. The field measurements were carried out in sixty three sounding stations, using collinear & symmetrical Schlumberger array, distributed on four traverses (NE-SW direction) across the axes of anticlines surrounding the study area. The maximum interval spacing between both current and potential electrodes has reached 843 and 90 meters consequently.

The final interpretation results were represented with four geoelectrical sections (as shown in the figure below). Continuous correlation with composite stratigraphic columns of surrounding anticlines was made and five electrical zones were distinguished reflecting the Soil, Quaternary deposits, upper & lower members of AlFatha Formation as well as Euphrates Formation.

This study has specified two strike slip faults affected by the Alpine orogeny. One transversal fault with NE-SW direction is located and clearly affects the true resistivity data and the anticlinal structures plunges in the east of the currently studying area. In addition a rotational strike slip fault has been specified, extending E-W parallel to the axes of the anticlines surrounding the area. The second fault is considered as an important tectonic element in the regional geology of the north of Iraq, this element divides between the Northern and the Southern blocks of Mosul, which is characterized by the existence of Taurus and Zagros trends consequently.

Keywords: geophysics, geotechnique, structure and geotectonic.

Değinilen Belgeler

Al-Mubarak, M.A. & Yuokhana, R.Y. (1976) Regional geological mapping of Al-Fatha- Mosul area. Geosurv. Baghdad, Unpub.

Bhattacharya, P.K. & Partra, H.P. (1968) Direct Current Geoelectric Sounding. Elsevier Publishing Company, Amsterdam, p. 135.

Geosurv. (1997) Composite lithological section of Alan & Atshan anticlines. State Organization for Minerals, Baghdad.

Keller, G.V. & Frischknecht, F.C. (1982) Electrical Methods in Geophysical Prospecting. 5th Edition, Pergamon press, New York, USA, p.519.

Mohi-Addin, R.M., Sissakian, V.K., Yousif, N.S., Amin, R.M. and Rofa, S.H. (1977) Report on the regional geological mapping of Mosul- Telafar area. Geosurv., Baghdad, Unpub.

Numan, N.M.S. & Al-Azzawi, N.K.B. (1993) Structural & geotectonic interpretation of vergence direction of anticlines in the forland folds of Iraq. AbhathAl-Yarmouk “Pure science and Engineering series”, Vol. 2, No. 2, p. 57-73.

Reynolds, J.M. (1998) An Introduction to Applied and Environmental Geophysics. Published wiley &sons. Ltd. England, p. 796.

Sharma, P.V. (1986) Geophysical Methods in Geology, Elsevier Scientific Publishing Company, Netherlands, p. 421.

Telford, W.M., Sherif, R.E., keys, D.A. (1991) Applied Geophysics, Cambridge University Press. p. 222 .

Vanovermeeren, R.A. (1989) Aquifer boundaries explored by geoelectrical measurements in the coastal plain of Yamen: Acase of equivalence. Geophysics. Vol. 54, No. 1, p. 38-48.

Oral presentation